## EXPERIMENT 7

AIM: Write a program to generate a square wave using µp 8086 interfaced with PPI8255 and execute the same.

THEORY:

8255 is a programmable peripheral interfacing IC which contains 3 I/O ports which can be programmed in different modes. To program the function to all three I/O ports,it contains a register called control register. The control register gives the signal which is used to define the function of each I/O port and in which mode they should operate.Port A, Port B and Port C are 8-bit ports. 8255 has direct bit select/reset compatibility which is available for Port C [BSRmode]. The ports can be operated in 3modes.

Mode 0- Simple I/O, Mode1-Strobbed I/O

Mode2-Strobbed bidirectional I/O.

# I/OmodeControl Word Format

The Bit Set/Reset (BSR) mode is applicable to port C only. Each line of port C (PC0 -PC7) can be set/reset by suitably loading the control word register. BSR mode and I/Omode are independent and selection of BSR mode does not affect the operation ofotherports inI/O mode.

| B3 | B2 | B1 | Bit/Pin of PortC Selected |
| --- | --- | --- | --- |
| 0 | 0 | 0 | PC0 |
| 0 | 0 | 1 | PC1 |
| 0 | 1 | 0 | PC2 |
| 0 | 1 | 1 | PC3 |
| 1 | 0 | 0 | PC4 |
| 1 | 0 | 1 | PC5 |
| 1 | 1 | 0 | PC6 |
| 1 | 1 | 1 | PC7 |

# ALGORITHM:

MAINPROGRAM

- Load Control Word Format of8255.

- Set the selected bit of port C.

- Call the delay subroutine.

- Reset the selected bit of port C.

- Call the delay subroutine.

- Terminate the program.

DELAY SUBROUTINE

- Load count in  BLregister.

- Load count in CLregister.

- Decrement count value in CL register.

- If CL≠0then goto stepno.3.

- Decrement count value in BL register.

- If BL≠0then gotostepno.2

Return to the main Program

Program :

CONCLUSION: Hence, we executed an ALP to generate a square wave using µp 8086 interfaced with PPI8255.

Name :Abdullah Khan

Roll No: 122A1002 --(02)

Batch :C1 Batch